This repository contains the MATLAB/Simulink implementation for the research work:

Design and Optimization of a Bidirectional Dual Active Bridge (DAB) DC-DC Converter using Dolphin Echolocation Algorithm for Microgrid Applications

The project demonstrates how a bio-inspired Dolphin Echolocation Algorithm (DEA) is used to automatically tune the PI controller gains of a phase-shift modulated DAB converter, resulting in superior transient performance compared to conventional manual tuning techniques.

📁 Repository Contents

This repo contains only the essential working files:

dab.slx        → MATLAB Simulink model of the bidirectional DAB converter

dolphin2.m    → MATLAB script implementing the Dolphin Echolocation Algorithm (DEA)

ResearchPaper.pdf → Complete research paper documenting theory & results

🎯 Project Objective

- Develop a state-space mathematical model of a bidirectional DAB converter suitable for microgrid energy storage interfaces.

- Implement Phase-Shift Modulation (PSM) based current control.

- Optimize PI controller gains (Kp, Ki) using DEA to minimize the Integral Absolute Error (IAE).

- Validate controller performance using MATLAB Simulink simulations for voltage regulation, power flow, and transient response.

⚙️ System Overview

Converter

- Topology: Dual Active Bridge (DAB) bidirectional isolated DC-DC converter

- Ports:

  - High-Voltage side: 1000 V DC link (PV / grid interface)

  - Low-Voltage side: 500 V storage bus (battery or ultracapacitor)

- Transformer turns ratio: 1:2

- Soft-switching (ZVS) operation achieved via Phase-Shift Modulation

Power Control

- Buck mode: HV → LV energy flow

- Boost mode: LV → HV energy flow

- Power is regulated using phase shift between primary and secondary bridge voltages.

🧠 Control Strategy

- Inner current control loop

- PI compensator generates phase shift command

- Feed-forward compensation for port voltage variations

- Phase shift ensures zero steady-state error and fast dynamic tracking.

🐬 Dolphin Echolocation Algorithm (DEA)

The script dolphin2.m implements DEA for controller tuning:

DEA Parameters

- Population size = 10

- Maximum iterations = 20

- Search ranges:

  - Kp ∈ [0, 10]

  - Ki ∈ [0, 5]

- Fitness function = Integral of Absolute Error (IAE)

Each dolphin represents a candidate PI gain pair. Simulations run inside the optimization loop and the best candidate is selected based on lowest IAE.

✅ Optimized Controller Gains

After 20 DEA iterations, the optimal gains obtained were:

Kp = 5.3558

Ki = 2.8991

These gains achieved:

- ✅ 56.5% faster settling time

- ✅ 98% overshoot reduction

- ✅ 45.8% lower IAE

- ✅ 50% lower ISE

- ✅ Significantly better voltage tracking vs manual tuning

(All results reported in ResearchPaper.pdf)

🚀 How to Run the Project

Step-1: Requirements

- MATLAB R2021a (or newer)

- Simulink

- Simscape Electrical / SimPowerSystems Toolbox

Step-2: Run Optimization

- Open MATLAB.

- Navigate to this repository folder.

- Open Simulink model:

open_system('dab.slx')

- Run DEA optimization:

run dolphin2.m

Step-3: What Happens

- DEA injects (Kp, Ki) candidates into the workspace as Kp_var and Ki_var.

- dab.slx runs simulations using these gains.

- Error signal is exported via a To Workspace block as error_signal.

- IAE is calculated and fed back into DEA.

- DEA converges to optimal PI gains and prints:

Optimal Kp = 5.3558

Optimal Ki = 2.8991

- Final waveforms and responses appear directly in Simulink scopes.

📊 Simulation Outputs

From the Simulink model and paper:

- Output voltage tracking with low overshoot & fast settling

- Primary and secondary bridge square-wave voltages with correct phase shift

- Continuous conduction inductor current waveform

- Input/output current regulation

- DEA convergence and controller error signals

📄 Research Paper

Full theoretical derivations, mathematical models, power equations, DEA flowcharts, simulation results, and performance comparisons are available in:

ResearchPaper.pdf

🔮 Future Work

- Experimental hardware validation using a scaled DAB prototype.

- Advanced control techniques:

  - Model Predictive Control (MPC)

  - Sliding Mode Control

- Multi-port microgrid converter extension.

- FPGA or DSP real-time controller implementation.

🔑 Keywords

Dual Active Bridge • Bidirectional DC-DC Converter • Microgrid • Phase Shift Modulation •
Dolphin Echolocation Algorithm • PI Optimization • MATLAB Simulink • Meta-heuristic Control